Processing Data from a Digital Level in Liscad

- To complete this tutorial you will require the Input/Output and Computations modules.

Aim

The aim of this tutorial is to show you how to import data from various systems into LISCAD. You will learn how to:

Import the observations from the digital level;

Process the data;

Store the new point elevations in the database.

Typical Scenario

A number of points were surveyed earlier using GPS. The point elevations did not have sufficient accuracy for use by subsequent traversing and adjustment; therefore these points were measured using a Leica DNA03 digital level.

The data of the DNA03 was transferred to disk into file: “Tutorial DNA.raw”.

Note that:

The procedure for processing data from Leica DNA03 or DNA10 as described below can also be applied to data from the Leica/Wild NA2000/3000 series instruments.

Before you begin

Select this button to install the required data files.

{button Install Tutorial Files, execfile(Seedatauk.exe,Tutorial.ctb;Tutorial DNA.see;Leica DNA.raw)}

Now proceed with the steps below.

- Select File/Open and open the file called "Tutorial DNA.see" in the Tutorial folder.

Use Display/Features – Points to enable the display of Crosses and Identifiers for all points
None of the points shown, except for point 1101, have an elevation.
The points without an elevation are shown in blue below.

- Select Task/Field Transfer

- Select Resolve/Create Field File and process the “Tutorial DNA.raw” file.
A warning will pop up because of formatting errors in the raw file.
The formatting errors are caused by data lines resulting from the adjustment of the data onboard the DNA.
These errors can be ignored.
The Field file will now look as follows:

- Next process the field file; select Resolve/Process Digital Level.
Select the field file we just created above:

- The Level Sheet Report is shown on screen giving the particulars of this level operation.
The level sheet only shows the actual data and does not attempt to make any adjustments.
The DNA03 and DNA10 can perform such adjustments in the field; Liscad will use the resulting elevations.



The report can be saved to a file or printed for future reference.

- At this stage we must indicate which points we want to update. Normally we will always update the points that have no elevations and leave alone the points that do.
This is reflected by the default settings of the dialog below.

- After selecting OK a short report is displayed telling which points have been updated and to which elevations.



 
It can be seen that point 3333, which was used to verify the field operation by staking it out near point 2, did not exist in the database and so is ignored by this action.

After the report window is closed, the screen will be updated and now all points will be shown in black indicating that an elevation is available.

Conclusion

You have now completed this tutorial and should have a good understanding of how to import data from digital levels and how to process it. Your data should appear as shown above.

You have also learnt how to:

- Control the display of point attributes.

- Store results into the database by updating the elevation of existing points.